CHAPTER 1

INTRODUCTION

# 1.1 Overview

Mobile robots are increasingly used in indoor environments for inspection, assistance, delivery, monitoring, and research. For a robot to be useful to a non-technical user, the interaction method should be simple and natural. Instead of requiring coordinates, joystick control, or programming commands, a user should be able to provide an instruction such as "Find the green marker" or, in later development, "Go to the door".

Natural-language mobile robot navigation is challenging because a human instruction must be converted into information that a robot can understand and execute. The robot must identify the intended target, connect the instruction with its current camera view, choose an appropriate movement action, and stop safely when the action is uncertain or an obstacle is detected.

This Final Year Project investigates how structured prompt engineering can support this process on one low-cost indoor mobile robot prototype. The finalized physical system uses a Raspberry Pi 4 for onboard prompt processing, USB-webcam perception, lightweight OpenCV visual grounding, action selection, and result logging. An ESP32 performs sensor reading, local safety checks, and four-motor control through two MX1508 motor-driver modules.

The first prototype intentionally targets the basic and repeatable goal find the green marker. It adapts the modular language-to-vision-to-action concept demonstrated by LM-Nav and the restricted prompt-based action-selection idea used by VLMnav. It does not attempt to reproduce a large autonomous navigation platform or run an unrestricted large vision-language model directly onboard.

# 1.2 Background and Motivation

Traditional mobile robot navigation commonly uses maps, localization, path planning, and sensor feedback. These methods are effective when the destination is represented by coordinates or predefined waypoints. However, ordinary users normally describe destinations using natural language and visual concepts.

Prompt engineering can constrain language interpretation into a structured response containing a target, landmarks, spatial relation, action goal, and uncertainty. Visual grounding can then connect the structured target to an image captured by the robot. For a physical robot, the output must also be restricted to approved actions and checked against local safety information.

The motivation for this project is therefore to investigate a practical division of responsibility. The Raspberry Pi 4 performs high-level language and visual processing, while the ESP32 performs time-sensitive sensor reading, motor commands, obstacle detection, and command-timeout safety. This architecture supports prompt-based robot navigation without allowing uncertain high-level output to bypass deterministic local safety rules.

# 1.3 Problem Statement

Natural-language instructions are intuitive for humans but are not directly usable by a mobile robot. Instructions may be ambiguous, incomplete, or dependent on objects visible in the environment. Free-form model responses are also difficult to connect reliably to motor control because they may contain unsupported actions or inconsistent wording.

Low-cost mobile robot platforms introduce further constraints. A USB webcam does not provide depth or a complete map. The Raspberry Pi 4 has limited resources for large vision-language models, so the first prototype requires a lightweight and measurable visual-grounding method. Two front ultrasonic sensors provide limited obstacle coverage, and the low-cost motor and power subsystems require careful integration.

There is therefore a need for a focused low-cost system that can convert a simple natural-language indoor goal into structured machine-readable information, ground a supported target using a webcam image, restrict the result to an approved action set, and validate the action using ESP32 safety information before movement.

# 1.4 Research Questions

1. How can structured prompt engineering convert simple indoor navigation instructions into consistent machine-readable targets, actions, and uncertainty?

2. How can Raspberry Pi 4 webcam-based visual grounding and ESP32 sensor feedback be combined to select and execute safe basic robot actions?

3. How reliably can the completed Raspberry Pi 4 prototype find and approach a supported visual landmark in a controlled indoor environment?

# 1.5 Aim and Objectives

The aim of this project is to design and implement a Raspberry Pi 4-based prompt-engineered mobile robot for basic navigation in a controlled indoor environment.

The objectives are:

1. To develop a structured prompt-engineering pipeline that converts a simple natural-language navigation instruction into fixed fields and an approved action set.

2. To integrate a Raspberry Pi 4, USB webcam, ESP32, two HC-SR04 sensors, GY-291 / ADXL345 accelerometer, two MX1508 motor drivers, four DC gear motors, and the completed protected power system into one physical robot.

3. To evaluate prompt validity, visual grounding, action correctness, latency, local safety behaviour, and simple physical navigation success.

# 1.6 Research Scope

The project is limited to one low-cost four-wheel indoor mobile robot using a Raspberry Pi 4 as the only onboard high-level compute platform. The robot operates in supervised controlled areas such as rooms, laboratories, and simple landmark test zones.

The first physical prototype focuses on the single-target goal find the green marker. The approved action set is limited to move_forward, turn_left, turn_right, search, and stop. The Raspberry Pi performs structured instruction processing, webcam capture, visual grounding, action selection, validation, and logging. The ESP32 reads the ultrasonic sensors and GY-291, controls four motors through two MX1508 modules, and enforces obstacle, sensor-fault, invalid-command, and command-timeout stop behaviour.

The GY-291 is used for X/Y/Z acceleration, gravity-based roll and pitch tilt, motion, vibration, and shock observations. It is not treated as a heading sensor. Advanced functions such as full simultaneous localization and mapping, precise coordinate navigation, wheel-encoder odometry, autonomous route planning, LiDAR, RGB-D perception, and unrestricted large-model control are outside the current scope.

A separate Docker-based laptop webcam demonstration may be used during FYP1 to illustrate expected action outputs before final physical testing. It is not a second physical project approach.

# 1.7 Significance of the Project

The project contributes a practical framework for connecting prompt engineering with a real low-cost robot. Structured output makes navigation decisions easier to parse, log, evaluate, and reject when unsafe. Separating Raspberry Pi high-level processing from ESP32 local control also demonstrates how model-based navigation can be combined with deterministic safety behaviour.

The first-prototype focus makes the research achievable and measurable. A reliable marker-navigation demonstration provides a baseline that can later be extended with object detection for chairs, doors, and signboards, relation-aware instructions, additional sensors, mapping, and stronger navigation models.

# 1.8 Organization of the Thesis

Chapter 1 introduces the project background, problem statement, research questions, objectives, scope, and significance. Chapter 2 reviews related work on language-guided navigation, prompt engineering, visual grounding, and suitable low-cost hardware. Chapter 3 presents the Raspberry Pi 4 methodology, system architecture, mechanical and electrical design, implementation procedure, and validation plan. Chapter 4 presents the expected FYP1 results and defines the physical measurements required during FYP2.

# 1.9 Summary

This chapter established the need for a focused and safety-aware approach to natural-language mobile robot navigation. The finalized project develops one Raspberry Pi 4 physical prototype with one ESP32 sensor, safety, and motor-control layer.