PROMPT ENGINEERING FOR

MOBILE ROBOT NAVIGATION

AMIR FARHAN BIN GHAFFAR

DEPARTMENT OF MECHATRONICS ENGINEERING  KULLIYAH OF ENGINEERING

INTERNATIONAL ISLAMIC UNIVERSITY MALAYSIA

JUNE 2026

PROMPT ENGINEERING FOR

MOBILE ROBOT NAVIGATION

AMIR FARHAN BIN GHAFFAR

Project Supervisor: Assoc. Prof. Ts. Dr. Hasan Firdaus Bin Mohd Zaki

A REPORT SUBMITTED IN PARTIAL FULFILMENT OF THE REQUIREMENT FOR A DEGREE OF BACHELOR OF MECHATRONICS ENGINEERING WITH HONOURS

ACKNOWLEDGEMENTS

I would like to express my sincere appreciation to my supervisor, Dr. Hasan Firdaus Bin Mohd Zaki, for his guidance, advice, and support throughout this Final Year Project. I am also thankful to the Kulliyyah of Engineering, Mechatronics Department, International Islamic University Malaysia (IIUM) for providing facilities and a good environment to carry out this project. Lastly, I would like to thank my friends who helped and supported me during the process, as well as my family for their encouragement and motivation.

LIST OF ABBREVIATIONS

IIUM		International Islamic University Malaysia

AUV		Autonomous Underwater Vehicle

BMS		Battery Management System

BoM		Bill of Materials

CB		Centre of Buoyancy

CAD		Computer-Aided Design

CG		Centre of Gravity

DC-DC	Direct Current to Direct Current (converter)

DOF		Degree(s) of Freedom

ESC		Electronic Speed Controller

FPS		Frames Per Second

GND		Ground (negative terminal)

IMU 		Inertial Measurement Unit

LAN 	Local Area Network

PWM		Pulse Width Modulation

ROV 	Remotely Operated Vehicle

SBC 		Single Board Computer

UDP 	User Datagram Protocol

USB 		Universal Serial Bus

VCC 	Positive Supply Voltage

CHAPTER 1

INTRODUCTION

# 1.1 Overview

Mobile robots are increasingly used in indoor environments for inspection, assistance, delivery, monitoring, and research. For a robot to be useful to a non-technical user, the interaction method should be simple and natural. Instead of requiring coordinates, joystick control, or programming commands, a user should be able to provide an instruction such as "Find the green marker" or, in later development, "Go to the door".

Natural-language mobile robot navigation is challenging because a human instruction must be converted into information that a robot can understand and execute. The robot must identify the intended target, connect the instruction with its current camera view, choose an appropriate movement action, and stop safely when the action is uncertain or an obstacle is detected.

This Final Year Project investigates how structured prompt engineering can support this process on one low-cost indoor mobile robot prototype. The finalized physical system uses a Raspberry Pi 4 for onboard prompt processing, USB-webcam perception, lightweight OpenCV visual grounding, action selection, and result logging. An ESP32 performs sensor reading, local safety checks, and four-motor control through two MX1508 motor-driver modules.

The first prototype intentionally targets the basic and repeatable goal find the green marker. It adapts the modular language-to-vision-to-action concept demonstrated by LM-Nav and the restricted prompt-based action-selection idea used by VLMnav. It does not attempt to reproduce a large autonomous navigation platform or run an unrestricted large vision-language model directly onboard.

# 1.2 Background and Motivation

Traditional mobile robot navigation commonly uses maps, localization, path planning, and sensor feedback. These methods are effective when the destination is represented by coordinates or predefined waypoints. However, ordinary users normally describe destinations using natural language and visual concepts.

Prompt engineering can constrain language interpretation into a structured response containing a target, landmarks, spatial relation, action goal, and uncertainty. Visual grounding can then connect the structured target to an image captured by the robot. For a physical robot, the output must also be restricted to approved actions and checked against local safety information.

The motivation for this project is therefore to investigate a practical division of responsibility. The Raspberry Pi 4 performs high-level language and visual processing, while the ESP32 performs time-sensitive sensor reading, motor commands, obstacle detection, and command-timeout safety. This architecture supports prompt-based robot navigation without allowing uncertain high-level output to bypass deterministic local safety rules.

# 1.3 Problem Statement

Natural-language instructions are intuitive for humans but are not directly usable by a mobile robot. Instructions may be ambiguous, incomplete, or dependent on objects visible in the environment. Free-form model responses are also difficult to connect reliably to motor control because they may contain unsupported actions or inconsistent wording.

Low-cost mobile robot platforms introduce further constraints. A USB webcam does not provide depth or a complete map. The Raspberry Pi 4 has limited resources for large vision-language models, so the first prototype requires a lightweight and measurable visual-grounding method. Two front ultrasonic sensors provide limited obstacle coverage, and the low-cost motor and power subsystems require careful integration.

There is therefore a need for a focused low-cost system that can convert a simple natural-language indoor goal into structured machine-readable information, ground a supported target using a webcam image, restrict the result to an approved action set, and validate the action using ESP32 safety information before movement.

# 1.4 Research Questions

1. How can structured prompt engineering convert simple indoor navigation instructions into consistent machine-readable targets, actions, and uncertainty?

2. How can Raspberry Pi 4 webcam-based visual grounding and ESP32 sensor feedback be combined to select and execute safe basic robot actions?

3. How reliably can the completed Raspberry Pi 4 prototype find and approach a supported visual landmark in a controlled indoor environment?

# 1.5 Aim and Objectives

The aim of this project is to design and implement a Raspberry Pi 4-based prompt-engineered mobile robot for basic navigation in a controlled indoor environment.

The objectives are:

1. To develop a structured prompt-engineering pipeline that converts a simple natural-language navigation instruction into fixed fields and an approved action set.

2. To integrate a Raspberry Pi 4, USB webcam, ESP32, two HC-SR04 sensors, GY-291 / ADXL345 accelerometer, two MX1508 motor drivers, four DC gear motors, and the completed protected power system into one physical robot.

3. To evaluate prompt validity, visual grounding, action correctness, latency, local safety behaviour, and simple physical navigation success.

# 1.6 Research Scope

The project is limited to one low-cost four-wheel indoor mobile robot using a Raspberry Pi 4 as the only onboard high-level compute platform. The robot operates in supervised controlled areas such as rooms, laboratories, and simple landmark test zones.

The first physical prototype focuses on the single-target goal find the green marker. The approved action set is limited to move_forward, turn_left, turn_right, search, and stop. The Raspberry Pi performs structured instruction processing, webcam capture, visual grounding, action selection, validation, and logging. The ESP32 reads the ultrasonic sensors and GY-291, controls four motors through two MX1508 modules, and enforces obstacle, sensor-fault, invalid-command, and command-timeout stop behaviour.

The GY-291 is used for X/Y/Z acceleration, gravity-based roll and pitch tilt, motion, vibration, and shock observations. It is not treated as a heading sensor. Advanced functions such as full simultaneous localization and mapping, precise coordinate navigation, wheel-encoder odometry, autonomous route planning, LiDAR, RGB-D perception, and unrestricted large-model control are outside the current scope.

A separate Docker-based laptop webcam demonstration may be used during FYP1 to illustrate expected action outputs before final physical testing. It is not a second physical project approach.

# 1.7 Significance of the Project

The project contributes a practical framework for connecting prompt engineering with a real low-cost robot. Structured output makes navigation decisions easier to parse, log, evaluate, and reject when unsafe. Separating Raspberry Pi high-level processing from ESP32 local control also demonstrates how model-based navigation can be combined with deterministic safety behaviour.

The first-prototype focus makes the research achievable and measurable. A reliable marker-navigation demonstration provides a baseline that can later be extended with object detection for chairs, doors, and signboards, relation-aware instructions, additional sensors, mapping, and stronger navigation models.

# 1.8 Organization of the Thesis

Chapter 1 introduces the project background, problem statement, research questions, objectives, scope, and significance. Chapter 2 reviews related work on language-guided navigation, prompt engineering, visual grounding, and suitable low-cost hardware. Chapter 3 presents the Raspberry Pi 4 methodology, system architecture, mechanical and electrical design, implementation procedure, and validation plan. Chapter 4 presents the expected FYP1 results and defines the physical measurements required during FYP2.

# 1.9 Summary

This chapter established the need for a focused and safety-aware approach to natural-language mobile robot navigation. The finalized project develops one Raspberry Pi 4 physical prototype with one ESP32 sensor, safety, and motor-control layer.

CHAPTER 2

LITERATURE REVIEW

# 2.1 Introduction

This chapter reviews literature related to Prompt Engineering for Mobile Robot Navigation. The project investigates how a mobile robot can receive a natural language instruction, extract useful navigation information, ground the instruction using visual input, and convert the result into simple movement commands. The work is positioned as a low-cost indoor Final Year Project (FYP) research prototype rather than an industry-level autonomous navigation system.

The main baseline paper for this project is LM-Nav: Robotic Navigation with Large Pre-Trained Models of Language, Vision, and Action by Shah et al. (2023a). LM-Nav is important because it presents a modular architecture for language-guided navigation. Its pipeline separates the problem into language understanding, visual grounding, and navigation execution. This structure is suitable for this FYP because the same idea can be adapted into one focused low-cost Raspberry Pi 4 robot with ESP32 low-level control.

The project does not attempt to reproduce the complete hardware capability of LM-Nav or newer large-scale navigation systems. Instead, the project adapts the core research idea into a student-level Raspberry Pi 4 physical robot. The first prototype uses the basic instruction "Find the green marker". The Raspberry Pi 4 produces structured navigation information, uses the USB-webcam image and lightweight OpenCV grounding, validates the proposed action using ESP32 sensor status, and sends a simple command such as move_forward, turn_left, turn_right, stop, or search to the ESP32 motor-control layer.

Several related works are reviewed to clarify the project scope. VLMnav is included as the main prompt and action-selection reference because it formulates navigation as a question-answering or action-choice task. VLMaps and HOV-SG are reviewed because they improve spatial grounding, but they require map construction, RGB-D sensing, or scene graph construction, so they are treated as future work rather than the main implementation. ViNT and NoMaD are reviewed as stronger navigation execution models, but they are not the main focus because this FYP emphasizes prompt engineering, visual grounding, and simple action control. More recent VLM and VLA navigation systems such as NaVid, Uni-NaVid, and NaVILA are also discussed as advanced research directions.

The aim of this chapter is therefore not only to describe related papers, but also to critically compare their suitability for a low-cost indoor robot prototype.

# 2.2 Mobile Robot Navigation and Natural Language Instructions

Mobile robot navigation normally requires a robot to perceive its environment, decide where to move, and execute motion safely. Traditional navigation systems often rely on a map, localization, path planning, obstacle detection, and low-level motor control. These systems are effective when the goal is specified in a robot-readable form, such as a coordinate, waypoint, or selected map location. However, this interaction style is not natural for non-technical users.

Natural language provides a more intuitive interface. A user can describe a goal using objects, places, and relations, for example "go to the door", "find the signboard", or "move toward the chair near the table". These instructions are simple for humans, but they are difficult for robots because they contain semantic meaning that must be connected to the physical environment.

Language-guided navigation therefore requires three main capabilities. First, the robot must understand the user's instruction. Second, it must ground the instruction in visual input or spatial knowledge. Third, it must convert the result into movement. This general process is shown in Figure 2.1.

Figure 2.1: General language-guided navigation process

For this FYP, the navigation problem is intentionally limited to indoor environments such as a corridor, room, laboratory, door area, table area, chair area, or signboard location. This limitation is necessary because the proposed hardware uses a USB webcam and simple wheeled motion, not LiDAR, RGB-D SLAM, or high-cost navigation hardware. The controlled environment makes it possible to evaluate the core research question: whether structured prompt engineering can produce useful navigation information for a simple mobile robot.

# 2.3 Prompt Engineering for Robot Navigation

Prompt engineering is the design of model instructions, examples, constraints, and output formats so that an LLM or VLM produces useful responses. In robot navigation, prompt engineering is important because the model output must be connected to a physical robot. A free-form explanation is not enough. The output should contain machine-readable information such as the target object, relevant landmarks, spatial relation, action goal, suggested action, and uncertainty level.

In an LM-Nav-style system, prompt engineering can be used to extract landmarks from a natural language command. In a VLMnav-style system, prompt engineering can ask a model to choose the best next action from a fixed set. For a low-cost robot prototype, both ideas are useful. The robot does not need a complex high-level planner in the first implementation if the prompt output can be converted into a small set of validated motor commands.

Table 2.1: Roles of prompt engineering in mobile robot navigation

| Role | Example Output | Purpose in This FYP |
| --- | --- | --- |
| Landmark extraction | door, chair, table, signboard | Identify objects or places mentioned by the user |
| Target selection | target: door | Decide the main navigation goal |
| Spatial relation extraction | chair near table | Preserve relations that affect grounding |
| Route or action interpretation | move_forward, turn_left | Convert instruction meaning into simple action options |
| Structured output formatting | JSON-style response | Allow Python code on the selected compute platform to parse the result |
| Uncertainty handling | uncertainty: high | Prevent unsafe execution when the model is unsure |
| Visual grounding query generation | door in indoor corridor | Improve matching between text and webcam image |

For this project, prompt engineering is treated as the interface between human language and robot action. The prompt must not only produce a correct sentence; it must produce an output that can be checked, logged, evaluated, and used by the onboard Raspberry Pi 4 decision logic.

# 2.4 Vision-Language Grounding for Indoor Landmarks

Vision-language grounding is the process of connecting text descriptions to visual observations. For mobile robot navigation, this means matching a target word such as "door", "chair", "table", "corridor", or "signboard" with what the robot sees through its camera.

CLIP and OpenCLIP-style models are common choices for image-text grounding because they embed images and text into a shared feature space. A text query can be compared with an image using cosine similarity:

score(image, text) = cosine_similarity(E_image(image), E_text(text))

where E_image is the image encoder and E_text is the text encoder. The image or region with the highest score can be treated as the most relevant match. A modern VLM may also be used by asking a question such as "Does this image contain a door?" or "Which action should the robot take to find the chair near the table?"

However, visual grounding remains a limitation for low-cost robots. A USB webcam provides only RGB images and does not directly provide depth, object distance, or a 3D map. The system may also struggle when multiple similar objects are visible, when lighting is poor, or when the target is partly outside the camera view. For this reason, the proposed project focuses on simple indoor landmarks and a small action set rather than full autonomous navigation in complex environments.

# 2.5 LM-Nav as the Main Baseline

Figure 2.2: LM-Nav real-world visual navigation platform (Shah et al., 2023a)

## 2.5.1 Overview of LM-Nav

LM-Nav: Robotic Navigation with Large Pre-Trained Models of Language, Vision, and Action by Shah et al. (2023a) is the main baseline for this project. LM-Nav addresses the problem that many visual navigation systems expect a goal image, while human users naturally provide language instructions. The paper avoids collecting a large language-annotated robot navigation dataset by composing pre-trained models.

The LM-Nav pipeline contains three main stages:

1. GPT-3 extracts landmarks from the natural language instruction.

2. CLIP grounds the extracted landmarks in visual observations.

3. ViNG navigates toward the selected visual goal.

This pipeline is shown in Figure 2.3.

Figure 2.3: LM-Nav baseline pipeline

## 2.5.2 Strength of LM-Nav

The main strength of LM-Nav is its modular architecture. It does not require one end-to-end model that directly maps language and images to motor commands. Instead, it separates language understanding, visual grounding, and action execution. This separation is useful for research because each module can be tested and improved independently.

LM-Nav is also important because it demonstrates that pre-trained models can be combined for robot navigation without training a new language-conditioned navigation model from scratch. This idea is suitable for an FYP because collecting a large robot dataset is not realistic within the time and cost constraints of a student project.

## 2.5.3 Limitation of LM-Nav

LM-Nav also has limitations that motivate this project. The language module mainly extracts landmarks. This may be insufficient for instructions that include spatial relations, route order, or constraints. For example, "move toward the chair near the table" should not be reduced only to chair and table; the relation chair near table is also important.

Another limitation is hardware and navigation complexity. LM-Nav was demonstrated with a more capable outdoor robot platform and a visual navigation model. A student FYP setup using a Raspberry Pi 4, ESP32 sensor and motor control, USB webcam, two ultrasonic sensors, and four DC motors cannot reproduce the full navigation capability directly. Therefore, this project uses LM-Nav as an architectural inspiration, not as a full implementation target.

## 2.5.4 Relevance to This Project

LM-Nav provides the central structure for the proposed prototype: language understanding, visual grounding, and action execution. The difference is that this project adapts the final execution stage into a simple action-control system. Instead of using a full visual navigation policy, the Raspberry Pi 4 chooses one of a small set of actions, checks obstacle status from the ESP32 ultrasonic module, and sends validated commands to the ESP32 motor-control layer. This makes the project realistic for a low-cost indoor robot while preserving the research value of structured prompt engineering.

# 2.6 VLMnav as a Prompt-Based Action Selection Reference

Figure 2.4: VLMnav prompt and action-selection interface (Goetting et al., 2024)

VLMnav: End-to-End Navigation with Vision Language Models by Goetting et al. (2024) is highly relevant because it formulates navigation as a question-answering problem. Instead of requiring the model to output a long navigation plan, the system can ask a VLM to choose among possible actions. This is close to the needs of the proposed robot because the first prototype only needs a small action set: move_forward, turn_left, turn_right, stop, and search.

The strength of VLMnav is that it connects visual observation, language instruction, and action selection through prompting. This supports the idea that prompt design can affect not only landmark extraction but also robot movement decisions. For example, a prompt can ask whether the current webcam image contains the target, whether the robot should turn to search, or whether it should stop because the target has been found.

The limitation is that direct VLM action selection may not be safe enough for unsupervised robot control. VLM outputs can be inconsistent, and the model may choose an action even when the target is not visible or the instruction is ambiguous. For this reason, the proposed FYP will validate the structured output, uncertainty level, model latency, and ESP32 ultrasonic obstacle status before physical motor execution. VLMnav is therefore treated as a prompt/action-selection reference, while the robot remains a controlled research prototype.

# 2.7 Spatial Grounding Methods: VLMaps and HOV-SG

Spatial grounding methods improve the connection between language and the physical environment. Two important examples are VLMaps and HOV-SG.

Figure 2.5: VLMaps language-grounded navigation example (Huang et al., 2023)

VLMaps: Visual Language Maps for Robot Navigation by Huang et al. (2023) builds an open-vocabulary visual-language map. Instead of matching a text query only to the current camera image, the robot can query a map that stores semantic visual features. The strength of VLMaps is that it provides spatial memory and can connect language to locations. Its limitation is that it requires map construction, camera pose information, and RGB-D or mapping support. This makes it more complex than the first version of the proposed FYP robot.

Figure 2.6: HOV-SG hierarchical navigation platform (Werby et al., 2024)

HOV-SG: Hierarchical Open-Vocabulary 3D Scene Graphs for Language-Grounded Robot Navigation by Werby et al. (2024) uses a hierarchical 3D scene graph to represent floors, rooms, and objects. This is powerful for long-horizon and multi-room navigation because the robot can reason about object and room relationships. However, it requires segmentation, 3D perception, graph construction, and more advanced sensing. This is not suitable for the first low-cost prototype, but it is valuable as a future improvement direction.

Both papers show that spatial grounding is important. They also highlight a gap for this project: a low-cost robot without RGB-D mapping still needs a way to preserve spatial relations from language. Structured prompt engineering can support this by extracting fields such as target, landmarks, and spatial_relation, even if the first implementation only uses webcam-based grounding.

# 2.8 Navigation Execution Models: ViNT and NoMaD

The execution stage determines how a robot physically moves toward a target. In advanced systems, this may involve a learned navigation policy, obstacle avoidance, and goal-conditioned control. ViNT and NoMaD are important because they represent stronger navigation execution methods than a simple rule-based motor command system.

Figure 2.7: ViNT visual navigation transformer architecture (Shah et al., 2023b)

ViNT: A Foundation Model for Visual Navigation by Shah et al. (2023b) proposes a visual navigation foundation model trained across diverse robot datasets. Its strength is generalization across platforms and environments. Its limitation for this FYP is that deploying and validating a full visual navigation model is outside the main scope. This project focuses on prompt engineering, visual grounding, and simple action control, not training or deploying a foundation navigation policy.

Figure 2.8: NoMaD visual navigation examples (Sridhar et al., 2023)

NoMaD: Goal Masked Diffusion Policies for Navigation and Exploration by Sridhar et al. (2023) uses a diffusion-based policy for goal-directed navigation and exploration. Its strength is stronger navigation behavior and exploration ability. Its limitation is that it requires a more complex policy deployment pipeline and suitable robot data. For the proposed project, NoMaD is therefore best treated as future work after the basic Raspberry Pi 4 physical robot pipeline is functioning.

These works are important because they show how the simple action-control layer in this FYP could later be replaced by a stronger navigation model. However, they are not the main implementation because the research focus is prompt-engineered interpretation of natural language instructions.

# 2.9 Recent VLM and VLA Navigation Systems

Recent work has moved toward more integrated VLM and VLA navigation systems. These systems can reason over video, language, and action more directly than the earlier modular LM-Nav design.

Figure 2.9: NaVid video-based navigation architecture (Zhang et al., 2024)

NaVid by Zhang et al. (2024) uses video-based VLM planning for vision-and-language navigation. Its strength is that it uses temporal visual context rather than a single image. This is useful for robot navigation because the current view alone may not contain enough information. Its limitation is that video-based reasoning and model inference can be expensive for a small embedded robot.

Figure 2.10: Uni-NaVid embodied navigation examples (Zhang et al., 2026)

Uni-NaVid by Zhang et al. (2026) extends the video-based approach toward a unified model for multiple embodied navigation tasks. Its strength is broad task coverage. Its limitation is that the system is more advanced and resource-intensive than a first FYP prototype.

Figure 2.11: NaVILA vision-language-action navigation examples (Cheng et al., 2025)

NaVILA by Cheng et al. (2025) studies VLA navigation for legged robots. Its strength is the connection between vision-language reasoning and real robot movement. Its limitation for this project is that it focuses on more complex legged platforms and server-style model pipelines, while the proposed robot is a wheeled low-cost indoor platform.

These papers are relevant because they show the direction of future navigation research. However, they also reinforce the need for a realistic baseline prototype. A student project can first demonstrate prompt-engineered instruction parsing, webcam grounding, and simple action control before attempting advanced VLA navigation.

# 2.10 Critical Comparison of Related Works

The reviewed papers differ significantly in method, hardware complexity, cost, and suitability for student-level implementation. Table 2.2 compares the papers based on their main method, strength, limitation, and relevance to this project.

Table 2.2: Critical comparison of related papers

| Paper | Main Method | Hardware or Platform | Strength | Limitation | Relevance to This Project |
| --- | --- | --- | --- | --- | --- |
| LM-Nav | GPT-3 landmark extraction, CLIP grounding, ViNG navigation | Real robot platform with RGB cameras and navigation sensors | Clear modular baseline for language to vision to action | Landmark-only extraction may lose spatial relations; full navigation setup is more complex than this FYP | Main baseline architecture adapted into a low-cost indoor prototype |
| VLMnav | VLM question-answering for action choice | Simulation-style embodied navigation setup with visual observations and action choices | Strong reference for prompt-based action selection | Direct VLM actions require validation before real robot control | Supports the proposed fixed action set and prompt design |
| VLMaps | Open-vocabulary visual-language maps | Mapping-based setup using RGB-D or pose-supported observations | Stronger spatial grounding and language-map querying | Requires map construction and additional sensing | Future improvement for map-based grounding |
| HOV-SG | Hierarchical open-vocabulary 3D scene graph | 3D perception, scene graph construction, and robot/simulator experiments | Supports object, room, and floor-level reasoning | Too complex for first low-cost webcam prototype | Future improvement for structured spatial memory |
| ViNT | Visual navigation foundation model | Multi-robot visual navigation datasets and deployment platforms | Stronger visual navigation execution | Does not focus on prompt engineering; deployment is beyond initial scope | Possible future replacement for simple action control |
| NoMaD | Diffusion-based goal-conditioned navigation policy | Robot navigation experiments with visual observations | Improves exploration and goal reaching | Requires navigation policy deployment and data support | Future work after baseline prototype |
| NaVid | Video-based VLM next-step planning | Simulation and real-world video navigation experiments | Uses temporal context for navigation decisions | More model and compute intensive than single-webcam baseline | Future direction for video-based reasoning |
| Uni-NaVid | Unified video-based VLA navigation model | Advanced embodied navigation platforms | Supports multiple navigation task types | Resource-intensive and not suitable as first implementation | Long-term improvement direction |
| NaVILA | Vision-language-action navigation for legged robots | Legged robot demonstrations and simulation benchmarks | Connects VLA reasoning to real movement | Focuses on complex legged platforms and larger model pipelines | Relevant conceptually but not the main hardware target |

The comparison shows that LM-Nav and VLMnav are the most directly relevant to this project. LM-Nav provides the modular structure, while VLMnav supports the idea of selecting actions through prompt-based reasoning. VLMaps, HOV-SG, ViNT, NoMaD, NaVid, Uni-NaVid, and NaVILA are important, but they require additional sensing, mapping, model deployment, or compute resources that are not realistic for the first prototype.

# 2.11 Hardware Suitability for This FYP

A critical issue for this project is hardware feasibility. Many research systems use expensive robots, RGB-D cameras, LiDAR, server GPUs, or advanced navigation policies. This FYP must use a lower-cost platform while still preserving a meaningful research contribution. Table 2.3 summarizes the suitability of literature components for the proposed implementation.

Table 2.3: Hardware and method suitability for this FYP

| Component or Method from Literature | Used in Original Paper or Related Work | Suitability for This FYP | Decision |
| --- | --- | --- | --- |
| RGB camera or webcam | Used in LM-Nav and many VLM navigation works as visual input | Highly suitable because it is low-cost and easy to connect through USB | An unbranded 1080p USB webcam will be used as the main visual sensor |
| Raspberry Pi 4 | Related to low-cost onboard Linux robot-compute platforms | Suitable for Python, OpenCV, USB devices, lightweight inference, and flexible development | Finalized and only physical compute board |
| ESP32 microcontroller | Not central in reviewed AI navigation papers, but common in low-cost sensor and motor integration | Suitable for reading ultrasonic sensors and controlling a motor driver independently from model inference | Will be used as the motor, sensor, and local safety controller |
| Two HC-SR04 ultrasonic sensors | Not the main perception method in the reviewed VLM papers | Suitable as a low-cost front-left and front-right proximity safety aid | Will be used for simple front obstacle awareness |
| GY-291 / ADXL345 accelerometer | Not the main perception method in the reviewed VLM papers | Suitable for three-axis acceleration, roll/pitch tilt, motion, vibration, and shock observations | Will support physical movement and chassis-condition monitoring |
| Two MX1508 dual motor drivers | Common low-cost motor-control hardware rather than a reviewed AI-navigation component | Suitable for controlling four small DC gear motors after current-rating verification | Will be used as the shared motor-driver subsystem |
| Four DC motors and chassis | Common in low-cost mobile robots but not the focus of high-level papers | Suitable for simple wheeled movement | Will be used for movement execution |
| 18650 battery pack | Standard power source for small mobile robot prototypes | Suitable for mobile operation when voltage and current requirements are handled safely | Will be used as the robot power source |
| RGB-D camera | Used or required by mapping-based works such as VLMaps and HOV-SG-style pipelines | Useful but increases cost and complexity | Optional future improvement |
| LiDAR | Used in some advanced robot navigation and scene understanding systems | Useful for safety and mapping but too costly for the first prototype | Not used in the baseline prototype |
| Full 3D scene graph | Used in HOV-SG and related spatial reasoning systems | Strong for spatial reasoning but requires 3D perception and mapping | Future improvement only |
| Large VLA model or server GPU | Used in advanced VLA systems such as NaVILA or Uni-NaVid-style pipelines | Powerful but outside the finalized onboard implementation scope | Future work only |
| CLIP, OpenCLIP, or VLM grounding | Used in LM-Nav-style grounding and VLM navigation | Suitable for prototype testing with indoor images | Will be used or evaluated depending on available compute |
| Learned navigation policy such as ViNT or NoMaD | Used in advanced navigation execution research | Useful but outside the main prompt-engineering scope | Future replacement for simple action control |

This table supports the design decision to build a low-cost indoor prototype. The project will not claim full autonomy in complex environments. Instead, it will demonstrate a baseline pipeline that future researchers can improve with better sensors, mapping, and navigation policies.

The positioning of the reviewed papers around the proposed baseline is shown in Figure 2.12.

Figure 2.12: Positioning of related works around LM-Nav

# 2.12 Research Gap and Motivation

The literature shows that language-guided robot navigation is an active research area, but several gaps remain for a low-cost FYP prototype.

First, existing systems can perform language-guided navigation, but many require expensive hardware, complex mapping, large models, or server-level compute. These systems are valuable research contributions, but they are not directly suitable for a student prototype using a Raspberry Pi 4, ESP32, USB webcam, two ultrasonic sensors, two MX1508 motor drivers, GY-291 accelerometer, and four DC gear motors.

Second, LM-Nav provides a strong modular baseline, but its language stage mainly extracts landmarks. Natural language instructions often include spatial relations and route information, such as "near the table", "turn left after the door", or "find the signboard". A low-cost prototype still needs to preserve this information, even if the first grounding module is simple.

Third, many advanced methods improve spatial grounding or navigation execution, but they do not directly answer how structured prompt engineering can support a simple indoor robot. VLMaps and HOV-SG show the value of maps and scene graphs, while ViNT and NoMaD show stronger navigation execution. However, this project begins earlier in the pipeline by asking how natural language can be converted into structured, validated robot action information.

Fourth, there is a need for a focused baseline physical prototype that future researchers can extend. A Raspberry Pi 4 robot with webcam grounding, ESP32 safety control, acceleration and tilt feedback, and simple actions provides a practical platform for later improvements such as RGB-D sensing, LiDAR, scene graph mapping, ViNT, NoMaD, or advanced VLM/VLA models.

Table 2.4: Research gap synthesis and project strategy

| Research Gap | Supporting Literature | Project Strategy |
| --- | --- | --- |
| Many language-guided navigation systems use costly or complex hardware | LM-Nav, HOV-SG, NaVILA, Uni-NaVid | Build one focused low-cost Raspberry Pi 4 prototype with ESP32 sensor, safety, and motor control |
| Landmark extraction alone may lose spatial relations and route information | LM-Nav, VLMaps, HOV-SG | Use structured prompt output with target, landmarks, spatial relation, action goal, suggested action, and uncertainty |
| VLM action selection needs validation before real robot execution | VLMnav | Check uncertainty, latency, approved actions, and ESP32 ultrasonic status before sending commands to the ESP32 motor-control layer |
| Mapping and scene-graph systems are useful but complex | VLMaps, HOV-SG | Treat RGB-D mapping and scene graphs as future work |
| Strong navigation policies are beyond the first prototype | ViNT, NoMaD | Start with simple action control and document future replacement options |

Based on these gaps, the project is motivated by the following research direction:

How can structured prompt engineering support visual grounding and simple action selection for a low-cost indoor mobile robot navigation prototype?

# 2.13 Chapter Summary

This chapter reviewed literature related to prompt engineering for mobile robot navigation. LM-Nav was identified as the main baseline because it provides a clear modular architecture of language understanding, visual grounding, and navigation execution. VLMnav was reviewed as the main prompt/action-selection reference because it frames navigation as a question-answering or action-choice task.

The chapter also reviewed VLMaps and HOV-SG as spatial grounding improvements, ViNT and NoMaD as stronger navigation execution models, and NaVid, Uni-NaVid, and NaVILA as advanced VLM/VLA navigation directions. These works show the potential of language-guided navigation, but they also reveal limitations related to cost, sensing requirements, compute resources, and implementation complexity.

The research gap is that there is a need for a low-cost indoor physical prototype that studies structured prompt engineering for simple mobile robot navigation. Chapter 3 explains how this project addresses the gap using one Raspberry Pi 4 implementation with an ESP32 motor, sensor, and safety layer.

CHAPTER 3

RESEARCH METHODOLOGY

# 3.1 Introduction

This chapter presents the finalized methodology for Prompt Engineering for Mobile Robot Navigation. The project contains one physical implementation: a Raspberry Pi 4 mobile robot with an ESP32 sensor, safety, and motor-control layer.

The methodology is inspired by the modular structure of LM-Nav and the prompt-based action-selection idea of VLMnav. Language interpretation, visual grounding, and navigation execution are separated so each stage can be tested. The first prototype targets find the green marker and restricts all output to the approved action set.

The finalized project contains one Raspberry Pi 4 physical robot approach.

- Raspberry Pi 4 performs onboard prompt processing, webcam processing, action selection, and logging.

- ESP32 performs sensor reading, deterministic safety, and four-motor control.

The robot uses a four-wheel chassis, USB webcam, two MX1508 motor drivers, adjustable motor buck converter, two HC-SR04 ultrasonic sensors, GY-291 / ADXL345 accelerometer, and the completed protected power system.

The intended action set is:

- move_forward

- turn_left

- turn_right

- stop

- search

The laptop-only Docker demonstration produces expected-results evidence before physical testing. It is not a second physical approach.

# 3.2 Research Design and Project Workflow

The project follows an iterative prototype methodology. The already-built mechanical and power systems are completed by wiring and validating the ESP32 sensor and motor layer, connecting Raspberry Pi USB serial, verifying webcam capture and OpenCV visual grounding, and testing one complete basic goal.

## 3.2.1 Methodology Alignment with Research Objectives

Table 3.1: Methodology Alignment with Research Objectives

| Research Objective | Method Used | Expected Output | Evaluation |
| --- | --- | --- | --- |
| Objective 1: Develop a structured prompt-engineering pipeline | Convert a simple instruction into fixed navigation fields and approved actions | Parseable target, action goal, and uncertainty | Prompt validity and target-extraction accuracy |
| Objective 2: Integrate one Raspberry Pi 4 physical robot | Connect Raspberry Pi, ESP32, webcam, sensors, MX1508 drivers, motors, and completed power system | Working end-to-end robot | Wiring, sensor, motor, USB serial, and power tests |
| Objective 3: Evaluate basic prompt-engineered navigation | Run repeated single-target indoor trials | Measured action, latency, safety, and movement results | Grounding correctness, action accuracy, safe-stop rate, and goal completion |

## 3.2.2 Overall Project Workflow

Figure 3.1: Proposed project workflow

## 3.2.3 Development Workflow

Table 3.2: Development Workflow

| Stage | Activity | Expected Output |
| --- | --- | --- |
| Stage 1 | Finalize the basic goal, action set, prompt schema, scenarios, and logging format | Common experimental definition |
| Stage 2 | Verify the completed power system and common-ground architecture | Stable compute and motor supplies |
| Stage 3 | Wire ESP32 sensors and two MX1508 drivers, then connect Raspberry Pi USB devices | Completed signal wiring |
| Stage 4 | Test ESP32 sensing, motor directions, timeout, and obstacle stop | Verified local safety and movement |
| Stage 5 | Install Raspberry Pi OS and verify webcam and USB serial | Working Raspberry Pi platform |
| Stage 6 | Integrate structured prompt processing, OpenCV grounding, action selection, and logging | Working end-to-end prototype |
| Stage 7 | Run repeated controlled trials and calculate metrics | Final physical results |

# 3.3 Finalized System Architecture

The Raspberry Pi 4 performs high-level processing. The ESP32 handles local real-time sensing, safety, and motor control. USB serial is used between the boards because it is simple to debug and leaves all ESP32 motor GPIO pins available.

Figure 3.2: Proposed system architecture

## 3.3.1 Hardware Roles

Table 3.3: Finalized Hardware Roles

| Component | Finalized Role | Main Interface | Main Strength | Main Constraint |
| --- | --- | --- | --- | --- |
| Raspberry Pi 4 | Onboard prompt processing, webcam capture, OpenCV grounding, action selection, and logging | USB webcam and USB serial | Flexible Linux and Python environment | Limited performance for large models |
| ESP32 | Sensor reading, deterministic safety, and four-motor control | USB serial and GPIO | Independent real-time safety layer | Runs only approved low-level commands |
| USB webcam | Front RGB image input | Raspberry Pi USB | Low-cost visual input | No dense depth or 3D map |
| Two HC-SR04 sensors | Front-left and front-right obstacle safety | ESP32 GPIO with Echo level shifting | Low-cost local obstacle detection | Limited coverage and accuracy |
| GY-291 / ADXL345 | Acceleration, roll/pitch tilt, motion, vibration, and shock observations | ESP32 I2C | Useful chassis-condition feedback | Does not provide yaw heading |
| Two MX1508 drivers | Four independent DC motor channels | ESP32 GPIO and regulated motor rail | Compact four-motor control | Requires current and thermal verification |
| Four DC gear motors | Physical forward, turn, search, and stop behaviour | MX1508 outputs | Simple four-wheel movement | No wheel-encoder feedback |

# 3.4 Requirements, Constraints, and Acceptance Criteria

Table 3.4: Requirements and Acceptance Criteria

| Requirement | Specification | Verification Method | Pass Criteria |
| --- | --- | --- | --- |
| Natural-language input | Accept a supported goal such as find the green marker | Input test | Instruction is accepted without manual rewriting |
| Structured prompt output | Return fixed machine-readable fields | JSON validation | Required fields are present and parseable |
| Raspberry Pi onboard processing | Run prompt parser, webcam, grounding, action selection, and logging onboard | Integration test | Raspberry Pi completes the navigation loop |
| Webcam visual grounding | Detect a supported coloured marker | Camera scenario test | Correct grounding or safe search response |
| Raspberry Pi-to-ESP32 USB serial | Exchange commands and sensor JSON at 115200 baud | Serial test | Stable command and status communication |
| Four-motor control | Two MX1508 modules control four motors | Raised-wheel test | Forward, left, right, search, and stop work |
| Local safety | Obstacle, sensor fault, timeout, or invalid command stops movement | Failure-case test | Unsafe movement is prevented |
| Power stability | Completed power system supplies stable rails | Voltage and runtime test | No unsafe voltage, reset, or severe drop |

The baseline does not include SLAM, LiDAR mapping, dense depth, or an unrestricted large vision-language model.

# 3.5 Data, Test Environment, and Experimental Materials

Table 3.5: First-Prototype Scenarios

| Scenario | Example Instruction | Expected Behaviour | Purpose |
| --- | --- | --- | --- |
| Supported target absent | "Find the green marker." | search | Verify safe visual search |
| Target on left | "Find the green marker." | turn_left | Verify left action selection |
| Target on right | "Find the green marker." | turn_right | Verify right action selection |
| Target centred and distant | "Find the green marker." | move_forward | Verify approach action |
| Target centred and close | "Find the green marker." | stop | Verify goal-stop behaviour |
| Unsupported or ambiguous target | "Go there." | stop | Verify prompt uncertainty handling |
| Obstacle present | "Find the green marker." | ESP32 forces stop | Verify independent local safety |

Table 3.6: Experimental Materials

| Material | Quantity or Role |
| --- | --- |
| Raspberry Pi 4 | Only onboard high-level compute board |
| ESP32 | Sensor, safety, USB serial, and motor-control controller |
| USB webcam | One front RGB camera |
| DC gear motor with wheel | Four motors and wheels |
| MX1508 dual motor-driver module | Two modules controlling four motors |
| Adjustable motor buck converter | Regulated motor-driver rail |
| HC-SR04 ultrasonic sensor | Two front obstacle sensors |
| GY-291 / ADXL345 accelerometer | Acceleration, roll/pitch tilt, motion, vibration, and shock sensing |
| Completed battery and power system | Mobile robot power source |
| Coloured marker | Repeatable first visual landmark |
| Instruction and scenario list | Common experimental inputs |
| JSONL result logs | Store prompts, detections, sensors, actions, latency, and failures |

Figure 3.3: Main hardware components for the finalized Raspberry Pi 4 robot

Table 3.7: Software and Model Requirements

| Item | Purpose | Finalized Tool or Example |
| --- | --- | --- |
| Raspberry Pi operating system | Run the onboard application | Raspberry Pi OS Lite 64-bit |
| High-level language | Prompt parser, grounding, validation, serial, and logging | Python |
| Webcam processing | Capture and process RGB frames | OpenCV |
| Prompt processing | Produce structured basic-goal output | Deterministic fixed-schema parser |
| Initial visual grounding | Detect supported coloured marker | OpenCV HSV segmentation |
| ESP32 firmware | Sensors, safety, USB serial, and two-MX1508 motor control | Arduino framework |
| Communication | Raspberry Pi to ESP32 commands and status | USB serial at 115200 baud |
| Result storage | Record experimental data | JSONL |
| Future grounding | Detect indoor objects and landmarks | OpenCV DNN, TensorFlow Lite, or API-based VLM |

# 3.6 Prompt Engineering Design

The first prototype uses a deterministic structured parser. This applies the main prompt-engineering principles required by the study: fixed fields, restricted actions, explicit uncertainty, and safe rejection of unsupported goals.

Table 3.8: Supported First-Prototype Prompt Inputs

| Prompt Input | First-Prototype Interpretation | Expected Result |
| --- | --- | --- |
| "Find the green marker." | Supported target and basic goal | Structured low-uncertainty output |
| "Go to the green marker." | Supported target and approach goal | Structured low-uncertainty output |
| "Go there." | Unsupported ambiguous target | High uncertainty and stop |
| Unsupported landmark instruction | Target outside first-prototype capability | High uncertainty and stop |

{
  "target": "door",
  "landmarks": ["door"],
  "spatial_relation": null,
  "action_goal": "find and move toward the door",
  "suggested_action": "move_forward",
  "uncertainty": "low"
}

Table 3.9: Structured Prompt Fields

| Field | Description | First-Prototype Example |
| --- | --- | --- |
| instruction | Original user instruction | find the green marker |
| target | Main supported target | green marker |
| landmarks | Important instruction objects | ["green marker"] |
| spatial_relation | Relation if supported | null |
| action_goal | Short interpreted goal | find and approach the green marker |
| uncertainty | Safety-oriented confidence label | low or high |

# 3.7 Webcam-Based Visual Grounding and Action Selection

The Raspberry Pi captures a USB-webcam frame and detects the selected marker using OpenCV HSV colour segmentation. The largest valid marker region is used for simple action selection.

Table 3.10: Visual Grounding and Action Decision Rules

| Condition | Selected Action | Reason |
| --- | --- | --- |
| Structured target unsupported or uncertain | stop | Prevent unsupported execution |
| Marker not detected | search | Rotate slowly to locate target |
| Marker centre is left of image centre zone | turn_left | Align camera with target |
| Marker centre is right of image centre zone | turn_right | Align camera with target |
| Marker is centred and below goal-size threshold | move_forward | Approach the target |
| Marker reaches configured image-area threshold | stop | Target is considered reached |
| ESP32 reports obstacle or sensor fault | stop | Local safety overrides high-level action |

# 3.8 Robot Action and Command Mapping

Table 3.11: Action Set and Motor Command Mapping

| Action | ESP32 Command | Four-Motor Behaviour |
| --- | --- | --- |
| move_forward | FWD | All motors move forward |
| turn_left | LEFT | Left motors reverse and right motors move forward |
| turn_right | RIGHT | Right motors reverse and left motors move forward |
| search | SEARCH | Robot rotates slowly |
| stop | STOP | All motor outputs stop |

The ESP32 accepts only the approved commands. Unknown commands, loss of both ultrasonic readings, and a command delay above 1,000 ms cause stop.

# 3.9 Baseline and Future Extension Boundaries

Table 3.12: Baseline and Future Extension Boundaries

| Criterion | Raspberry Pi 4 Baseline | Future Extension |
| --- | --- | --- |
| Physical implementation | One Raspberry Pi 4 and ESP32 robot | Retain the same safe robot base |
| Prompt processing | Deterministic fixed-schema parser | Lightweight local model or API-based VLM |
| Visual grounding | OpenCV HSV coloured-marker detection | Object detector or vision-language grounding |
| Navigation action | Restricted five-action set | Relation-aware planning or learned policy |
| Position feedback | Ultrasonic and GY-291 observations | Wheel encoders, magnetometer, or full IMU |
| Environment model | Current RGB frame only | RGB-D, LiDAR, mapping, or scene graph |
| Result role | Final physical robot results | More advanced FYP2 or future research |

# 3.10 Mechanical Design and Physical Integration

The finalized Fusion 360 robot design provides mounting positions for the Raspberry Pi 4, ESP32, USB webcam, two front ultrasonic sensors, GY-291, two MX1508 drivers, completed power system, and four motors.

Figure 3.4: Finalized Fusion 360 physical robot model

The existing render records the physical design and Raspberry Pi 4 mounting layout.

Figure 3.5: Finalized Fusion 360 robot multi-view layout

The front, side, rear, and top views support sensor visibility, wheel clearance, component spacing, cable access, and Raspberry Pi 4 mounting verification.

Figure 3.6: Finalized mechanical design sketch and dimensions

The dimensioned design sketch records an approximate overall length of 232.24 mm, width of 188.91 mm, and height of 129.57 mm. These dimensions guide fabrication, mounting, and clearance checks before physical assembly.

## 3.10.1 Custom-Designed and Fabricated Components

The robot design includes modular printable parts for the Raspberry Pi 4, ESP32, battery system, motor drivers, GY-291, webcam, and both ultrasonic sensors. These mounts organize components, maintain sensor visibility, and reduce movement during operation.

The STL files are stored in the project STL/ directory so that damaged or revised mounts can be reproduced without redesigning the complete chassis.

## 3.10.2 Finalized Circuit Architecture

An L298N is a motor driver rather than a buck converter. The finalized architecture therefore uses one separate adjustable motor buck converter and two MX1508 dual motor-driver modules.

Figure 3.7: Finalized component-based circuit architecture

The power-bank module must explicitly support a 3S lithium-ion pack and the required continuous current. A common single-cell power-bank board must not be connected directly to the 3S pack. HC-SR04 Echo signals must be level-shifted before entering ESP32 GPIO.

Figure 3.8: Finalized ESP32 sensor and motor wiring

# 3.11 Company, Software, and Tool Involvement

Table 3.13: Company, Component, and Software Involvement

| Component or Tool | Company or Organization | Involvement |
| --- | --- | --- |
| Raspberry Pi 4 | Raspberry Pi Ltd | Only onboard high-level compute board |
| ESP32 | Espressif Systems | Low-level sensing, safety, and motor control |
| GY-291 / ADXL345 | Analog Devices device on third-party module | Acceleration, roll/pitch tilt, motion, vibration, and shock sensing |
| MX1508 modules | Supplier to be confirmed | Four-motor driver subsystem |
| Autodesk Fusion 360 | Autodesk | Mechanical design |
| Python | Python Software Foundation | High-level software |
| OpenCV | OpenCV project | Webcam processing and marker grounding |
| Arduino framework | Arduino project / Espressif ecosystem | ESP32 firmware |
| Docker | Docker, Inc. | Isolated laptop expected-results demonstration |
| diagrams.net / draw.io | JGraph / diagrams.net | Report diagrams |

# 3.12 Software Implementation and Coding

Table 3.14: Source Code and Documentation Modules

| File or Module | Target | Purpose | Status |
| --- | --- | --- | --- |
| docs/raspberry_pi_4_full_software_setup.md | Documentation | Raspberry Pi OS-to-final-result setup | Provided |
| docs/circuit_wiring_guide.md | Documentation | Exact Raspberry Pi, ESP32, sensor, and MX1508 wiring | Provided |
| docs/laptop_only_expected_results_setup.md | Documentation | Separate Docker-based FYP1 expected-results demonstration | Provided |
| src/raspberry_pi_robot/robot_controller.py | Raspberry Pi 4 | Prompt parser, OpenCV grounding, action selection, USB serial, and logging | Provided |
| src/laptop_expected_results/web_app.py | Docker laptop demo | Browser-webcam frame processing and expected action output | Provided |
| firmware/esp32_robot_controller/esp32_robot_controller.ino | ESP32 | USB serial, sensors, safety, and four-motor control | Provided |

# 3.13 Equations and Decision Rules

Table 3.15: Main Equations and Decision Rules

| Equation or Rule | Description | Evaluation Use |
| --- | --- | --- |
| d_cm = t_echo_us / 58 | HC-SR04 distance approximation | Convert Echo duration to centimetres |
| Prompt Validity = N_valid / N_total * 100 percent | Valid structured-output percentage | Prompt reliability |
| Accuracy = N_correct / N_total * 100 percent | General correctness percentage | Grounding and action metrics |
| T_total = T_prompt + T_capture + T_grounding + T_sensor + T_comm | End-to-end response time | Raspberry Pi latency |
| a_exec = stop for invalid output, high uncertainty, obstacle, sensor fault, timeout, or invalid action | Safety decision rule | Safe-failure evaluation |
| Movement Success Rate = N_success / N_scenarios * 100 percent | Successful simple-goal percentage | Physical robot behaviour |
| a_mag = sqrt(a_x^2 + a_y^2 + a_z^2) | Total measured acceleration magnitude | Motion, vibration, and shock observation |

# 3.14 Implementation Procedure

Table 3.16: Implementation Procedure

| Stage | Activity | Verification | Expected Result |
| --- | --- | --- | --- |
| Stage 1 | Verify completed power rails and common ground | Multimeter and load test | Stable compute and motor supplies |
| Stage 2 | Wire ESP32, sensors, and both MX1508 drivers | Continuity and pin-map review | Correct signal wiring |
| Stage 3 | Flash ESP32 and connect it to Raspberry Pi by USB | PING and sensor-status test | Stable commands and JSON status |
| Stage 4 | Test four motors with wheels raised | Individual and combined action tests | Correct movement directions |
| Stage 5 | Install Raspberry Pi OS and test webcam | Camera capture test | Working Raspberry Pi platform |
| Stage 6 | Run Raspberry Pi controller without --execute | Dry-run logs | Correct prompt, grounding, action, and STOP command |
| Stage 7 | Enable supervised raised-wheel movement | End-to-end command test | Correct action-to-motor behaviour |
| Stage 8 | Run controlled floor scenarios | Scenario checklist and logs | Measured basic-goal results |

# 3.15 Testing and Validation Plan

Table 3.17: Testing and Validation Matrix

| Test Stage | Test Setup | Metric | Pass Criteria |
| --- | --- | --- | --- |
| Prompt format test | Supported and unsupported instructions | Prompt output validity | Required fields parse correctly |
| Webcam test | USB webcam on Raspberry Pi | Frame capture success | Reliable usable frames |
| Grounding test | Supported marker in controlled views | Grounding correctness | Correct match or safe search |
| USB serial test | Raspberry Pi and ESP32 | Command/status reliability | Repeated valid exchanges |
| Ultrasonic test | Two front sensors | Distance availability and error | Both sensors report usable values |
| GY-291 test | Stationary, tilted, moved, and lightly vibrated robot | Plausibility and response | Expected changes are observable |
| Motor test | Two MX1508 modules and four motors | Movement correctness | All approved actions work |
| Safety test | Obstacle, sensor fault, timeout, invalid command, and uncertainty | Safe-stop rate | Robot stops safely |
| Full scenario test | Controlled green marker | Goal completion | Intended basic behaviour is completed |

# 3.16 Evaluation Metrics

Table 3.18: Evaluation Metrics

| Metric | Description |
| --- | --- |
| Prompt output validity | Percentage of outputs containing all required fields |
| Supported-target extraction accuracy | Percentage of correctly extracted supported targets |
| Visual grounding correctness | Percentage of correctly grounded marker conditions |
| Action-selection accuracy | Percentage of expected selected actions |
| End-to-end latency | Time from instruction and frame input to validated action |
| USB serial reliability | Percentage of valid command and sensor-status exchanges |
| Safe-stop rate | Percentage of unsafe cases correctly stopped |
| Movement success rate | Percentage of completed simple-goal scenarios |
| GY-291 response | Plausibility of acceleration, tilt, motion, and vibration observations |
| Power stability | Resets, voltage drops, or failures during testing |

# 3.17 Safety, Limitations, and Future Work

Testing must use the completed protected power system, fuse, reachable main switch, correct regulated rails, and a common ground. Motor tests begin with the robot raised. HC-SR04 Echo pins require level shifting. During the first prototype, the ESP32 is powered only by the Raspberry Pi USB cable.

The robot stops for invalid output, high uncertainty, obstacle detection, sensor fault, unknown commands, or command timeout. Testing is supervised in a controlled indoor area, and webcam testing avoids recording identifiable people without permission.

The USB webcam does not provide dense depth or a 3D map. Two ultrasonic sensors provide limited obstacle coverage. The GY-291 does not provide yaw heading. The Raspberry Pi 4 has limited performance for large models, and the first prototype supports only simple coloured-marker goals.

Future work may include lightweight object detection, API-based VLM experiments, wheel encoders, magnetometer or full IMU, RGB-D camera, LiDAR, mapping, relation-aware prompts, and stronger navigation policies.

# 3.18 Summary

This chapter finalized the methodology around one Raspberry Pi 4 physical robot. The project connects structured prompt processing and OpenCV visual grounding to an ESP32 that independently handles sensors, safety, and four-motor control.

The first measurable target is a reliable and safe single-goal indoor marker-navigation demonstration.

CHAPTER 4

EXPECTED RESULTS AND DISCUSSION

# 4.1 Introduction

This chapter presents the expected results for FYP1. The expectations are based on the finalized Raspberry Pi 4 architecture, structured prompt design, completed physical hardware, ESP32 safety logic, Docker laptop expected-results demonstration, and planned physical validation. Final measured robot results will be presented during FYP2.

# 4.2 Expected Basic Prototype Result

The first Raspberry Pi 4 prototype is expected to complete the simple and repeatable goal find the green marker. The instruction should be converted into a structured target and approved action set. OpenCV colour grounding should locate the marker in the webcam view and select an action from its image position.

Table 4.1: Summary of Expected Basic Prototype Results

| Condition | Expected Result |
| --- | --- |
| Clear supported marker instruction | Produces a valid structured target and low uncertainty |
| Unsupported or unclear instruction | Stops safely |
| Marker not visible | Selects search |
| Marker visible on left or right | Selects the corresponding turn action |
| Marker centred and not close | Selects move_forward |
| Marker reaches configured goal size | Selects stop |
| Ultrasonic obstacle, sensor fault, or command timeout | ESP32 forces stop |

# 4.3 Expected Docker Laptop Demonstration

The laptop-only Docker demonstration is expected to show the same planned action interface before physical robot testing. A browser captures a coloured marker through the laptop webcam and sends frames to the containerized Python/OpenCV service. The application displays TURN LEFT, TURN RIGHT, MOVE FORWARD, SEARCH, or STOP, and the user manually moves the laptop according to the displayed action.

Table 4.2: Expected Raspberry Pi Physical and Docker Demonstration Roles

| Area | Raspberry Pi 4 Physical Prototype | Laptop Docker Expected-Results Demonstration |
| --- | --- | --- |
| Camera input | Robot-mounted USB webcam | Laptop browser webcam |
| Processing | Onboard Python and OpenCV | Containerized Flask, Python, OpenCV, and NumPy |
| Action output | Validated command sent to ESP32 | Instruction displayed on screen for manual movement |
| Result status | Final physical robot result during FYP2 | Expected-results evidence only |

This demonstration provides expected-results evidence only. It is not a second physical implementation and is not used as the final robot result.

# 4.4 Expected Safety Simulations

The ESP32 uses the minimum reading from the two front HC-SR04 sensors. With the planned 25 cm threshold, the robot is expected to stop when either valid sensor reading is below 25 cm.

Figure 4.1: Expected obstacle-stop response at the planned 25 cm threshold

The ESP32 also applies a 1,000 ms command timeout. If no new valid command is received within this period, all motor outputs are expected to stop.

Figure 4.2: Expected motor response when the command timeout exceeds 1,000 ms

These simulations demonstrate the intended decision logic only. The final distance threshold and physical stopping response require calibration and repeated testing during FYP2.

# 4.5 Required FYP2 Results

Table 4.3: Required FYP2 Measurements

| Evaluation Area | Required Measurement |
| --- | --- |
| Prompt engineering | Structured output validity and supported-target extraction accuracy |
| Visual grounding | Marker-grounding correctness and safe missing-target behaviour |
| Raspberry Pi performance | OpenCV processing and end-to-end action latency |
| USB communication | Command and sensor-status reliability |
| Sensors and safety | Ultrasonic error, GY-291 response, timeout, and safe-stop success |
| Robot movement | Action correctness and simple-goal completion rate |
| Power system | Voltage stability, resets, current, and operating duration |

# 4.6 Discussion

The expected results suggest that structured prompt engineering can provide a simple interface between a natural-language goal and a restricted robot action set. The Raspberry Pi 4 is expected to provide a flexible environment for Python, OpenCV, USB devices, logging, and later model experiments, while the ESP32 independently prevents movement during obstacle, sensor-fault, timeout, and invalid-command conditions.

The basic marker goal is intentionally limited. Demonstrating it reliably is more valuable than claiming complex navigation without sufficient sensor coverage, mapping, or model capability. Once the baseline is verified, the project can extend to object detection for indoor landmarks and more complex instructions.

# 4.7 Summary

This chapter presented the expected FYP1 outcomes for the Raspberry Pi 4 prototype, Docker laptop expected-results demonstration, and safety logic. Final physical results will measure whether the robot can safely find and approach a supported visual landmark.

# REFERENCE

Cheng, A.-C., Ji, Y., Yang, Z., Gongye, Z., Zou, X., Kautz, J., Biyik, E., Yin, H., Liu, S., & Wang, X. (2025). NaVILA: Legged robot vision-language-action model for navigation. *Proceedings of Robotics: Science and Systems*. https://doi.org/10.15607/RSS.2025.XXI.018

Goetting, D., Singh, H. G., & Loquercio, A. (2024). *End-to-end navigation with vision language models: Transforming spatial reasoning into question-answering*. arXiv. https://doi.org/10.48550/arXiv.2411.05755

Huang, C., Mees, O., Zeng, A., & Burgard, W. (2023). Visual language maps for robot navigation. *Proceedings of the IEEE International Conference on Robotics and Automation (ICRA)*. https://vlmaps.github.io/

Shah, D., Osinski, B., Ichter, B., & Levine, S. (2023a). LM-Nav: Robotic navigation with large pre-trained models of language, vision, and action. *Proceedings of the 6th Conference on Robot Learning, Proceedings of Machine Learning Research, 205*, 492-504. https://proceedings.mlr.press/v205/shah23b.html

Shah, D., Sridhar, A., Dashora, N., Stachowicz, K., Black, K., Hirose, N., & Levine, S. (2023b). ViNT: A foundation model for visual navigation. *Proceedings of the 7th Conference on Robot Learning, Proceedings of Machine Learning Research, 229*, 711-733. https://proceedings.mlr.press/v229/shah23a.html

Sridhar, A., Shah, D., Glossop, C., & Levine, S. (2023). *NoMaD: Goal masked diffusion policies for navigation and exploration*. arXiv. https://arxiv.org/abs/2310.07896

Werby, A., Huang, C., Buchner, M., Valada, A., & Burgard, W. (2024). Hierarchical open-vocabulary 3D scene graphs for language-grounded robot navigation. *Proceedings of Robotics: Science and Systems*. https://doi.org/10.15607/RSS.2024.XX.077

Zhang, J., Wang, K., Xu, R., Zhou, G., Hong, Y., Fang, X., Wu, Q., Zhang, Z., & Wang, H. (2024). NaVid: Video-based VLM plans the next step for vision-and-language navigation. *Proceedings of Robotics: Science and Systems*. https://doi.org/10.15607/RSS.2024.XX.079

Zhang, J., Wang, K., Wang, S., Li, M., Liu, H., Wei, S., Wang, Z., Zhang, Z., & Wang, H. (2026). Uni-NaVid: A video-based vision-language-action model for unifying embodied navigation tasks. *Robotics: Science and Systems 2026 Program*. https://roboticsconference.org/program/papers/13/